ALBIUS: Achievements in reporting period: April 1st 2011 until June 30th 2011

> Please mention:

>

> - List of working documents

http://www.radionet-eu.org/fp7wiki/doku.php?id=jra:albius:processing

(as reported June 30, 2010)

> - Main Activities during

- EVN OH observation of Cyg A, NGC 1068 (E10C014) for RFI mitigation test

Correlation was completed:

Correlator:

Bonn DiFX 2 correlator extended with multi-rate filter for RFI mitigation

Multi-rate filter topology:

Arbitrary configurable chain of different filter types

Filters implemented:

Decimator block,

IIR SOS biquad filter block (efficient lowpass),

DSVF digital state variable filter block (most efficient lowpass),

Integrator block,

Moving average block.

Two correlation passes:

one with RFI mitigation filter switched on,

one with filter switched off.

Calibration is advanced:

Cyg A data have been calibrated in AIPS and first cube produced.

Continuum image is good.

Spectral baselines need still some improvement to look for OH absorption.

Next:

Can begin comparing spectra with and without RFI mitigation to

validate mitigation effectiveness.

- Focal Plane Array RFI mitigation:

Added specification to APERTIF beamformer design for Effelsberg

for the formation of cross-correlations between elements to enable

use of eigen decomposition methods.

G. Knittel (MPIfR) has implemented cross-correlation computation in

his Effelsberg APERTIF demo beamformer.

Developed Beamformer C++ library.

Partially under work.

Library uses LAPACK/ATLAS via Armadillo C++ (Linux, Windows) for

fast matrix computation.

Currently in DiFX SVN repository.

To be uploaded onto project wiki.

Implemented in Matlab:

several RFI cancellation algorithms,

RFI detection,

two FPA model generators,

UV gridder,

various testing code.

Implemented in final C++ library:

decompositions, RFI detection, nulling, subtraction algorithms

Included in DiFX SVN due to great potential for DiFX phase array processing.

Algorithm 1:

Post-correlation adaptive filter using eigen-decomposition of

the covariance matrix (for example Briggs & Kocz)

Algorithm 2:

Post-correlation RFI templating and subtraction using covariance

matrix with one or more elements being RFI reference antennas

(for example Kesteven, Briggs or van der Veen)

Test Data:

Synthetic data with and without reference antennas generated for

controlled testing of the algorithms, and

Westerbork APERTIF correlation matrices from Wim van Capellen.

Results on synthetic data:

The implemented subspace nulling (Matlab, C++) is highly effective

in cancelling interferers in the data models (> 60 dB).

The implemented subtraction (Matlab, C++) is equally effective for

the data models, within certain constraints on the number of

available reference antennas in practice and the maximum number

of interferers.

Results on Virgo A (APERTIF data):

Strong RFI at 1490 MHz from Afristar can be suppressed enough that

the source (Virgo A) can then be seen in the central beam.

Cancellation is reasonably good but not as excellent as seen

in the synthetic test data nor as good as demonstrated by

Briggs & Kocz (2005), probably since they used reference

antennas to get good interference-to-noise ratio on the

interferer, which is not available in the APERTIF data.

Figures:

The following are two figures with captions to consider.

Figure 1: virgoA_RonRoff_channelAutoCorrs_orig_vs_nulled.png

Caption: 3D surface plot of APERTIF element autocorrelation power spectra

before (left) and after (right) RFI mitigation, showing that the

source (Virgo A) becomes visible after RFI mitigation.

Axis 1 sloping 45 degrees upwards towards left: APERTIF array

element number.

Axis 2 sloping 45 degrees upwards towards right: frequency channel

(1 to 72 = 1483.0 MHz to 1496.7 MHz spaced 195.3125 kHz)

Axis 3 vertical: power difference between on-source and off-source pointings

Strong RFI is present from Afristar at 1490 MHz, visible before

mitigation as the rumble through the middle of the spectrum

in all elements (blue-green maxima sloping upwards to the left).

After mitigation the rumble of RFI is much reduced, and the source

(Virgo A) is visible in array element 31 (orange ridge sloping

upwards to the right).  A strong peak becomes visible in

element 11 channel 17 (1486.2 MHz) after mitigation, it is present

in the mitigated off-source spectrum and not in the mitigated on-source

spectrum. Its origin is presently unclear, and might be an

algorithm artefact.

Figure 2: virgoA_RonRoff_nulled_centerelement_equiscale.png

Caption: Total power spectra for the central APERTIF element

(number 31), differenced between on-source and off-source,

before and after RFI mitigation (left and right respectively).

The source is Virgo A.

Horizontal axis is frequency channel (1 to 72 = 1483.0 MHz to 1496.7 MHz

spaced 195.3125 kHz), vertical axis is power difference.

RFI from Afristar at 1490 MHz is visible in the centre of the spectrum.

The RFI is mitigated quite well in the right hand spectrum.

The remaining sinusoidal spectral baseline is due to known

standing wave in the Westerbork dish.

Remaining issues:

- Why is quality of mitigation limited?  Need reference antenna?

- Too many interferers?

Source not seen by other elements hence UV gridding fails?

- Can we deal with spectral baseline? (should be correlated between

elements)

- DiFX has beamformer mode for use with focal-plane arrays and phase arrays,

could in principle integrate calls to the C++ Beamformer library

there, such as enabling Nulling or beamformer adaptive weight updates.

> - Meetings attended by people involved

None.

> - Problems / Issues

None.

> - Red flags - major problems

None.

> - Forward Look

> - Expected Milestones

Delivery of all deliverables is expected by end of 2011.

> - Number of Persons working in JRA

One person - Jan Wagner. (plus a small fraction of Alan Roy)